SLIDING BOLLARDS

(HEALD Matador 6)

- GENERAL

      - SUMMARY

        - The Heald Matador 6 offers an unmatched combination of IWA-14 tested security, manual operation and flush/surface mount installation. It is the only surface mount sliding bollard system available tested to halt a 16,800 lb. truck travelling at 40 mph. Easily deployable, the unit’s bollard height is 44.8", its base is 321.6" L x 84’ W and 4.3’’ H. Fitting is extremely cost effective and efficient, as the Matador 6 can be surface mounted or alternatively can be fitted flush with the road surface requiring only minimal concrete and no pre-casting of the pit. Excavations are 84" W x 321.6" L and only 5.90" D.  Work Included:  Provide labor, materials and equipment necessary to complete the work of this Section, including but not limited to the following:

      - SUBMITTALS

        - A. Reports and Certifications:  Submit the following:

- 1. Crash Test Report: Provide a copy of the crash test report summary, from an independent, ISO 17025 accredited test facility, showing assignment of the IWA14-1:2003 Bollard V/7200[N2A]/64/90:0.1 rating. (DOS K8/M40/P1  equivalent)

- 2. Crash Test Certification:  Provide copies as required.

        - Operations and Maintenance Manuals: Submit at least two copies of operating and maintenance instructions at least two weeks prior to installation.

- 1. Operating Instructions:  Include step by step procedures for operation.  Also include the manufacturer’s name, manufacturer’s contact information, model number, parts lists, and brief description of all equipment and their basic operating features.

- 2. Maintenance Instructions: Include maintenance schedule, routine maintenance procedures, troubleshooting and repair procedures, and spare parts list.

        - Warranty:  Submit executed copy of manufacturer’s warranty.

      - QUALITY ASSURANCE

        - Installation services will be provided by one of the Manufacturer’s Certified Physical Security Contractor’s (PSC).  The PSC will have:

          - Documentation from the product manufacturer(s), dated within the past year, that they are a certified installer and have been properly trained on the equipment to be installed.

          - A minimum of 10 years experience in the installation, setup, adjustment, and service of the type of equipment supplied;

          - Experience in at least 3 similar cost-sized projects and provide documented owner satisfaction of the project results;

          - Paydex equal to or greater than the industry median.

          - PSC must be capable of bonding projects up to $3 million and must demonstrate such with a letter from a reputable bonding entity with bid.

          - PSC must provide documentation with bid indicating that the company has in place at least $2 million in general liability coverage as well as at least $1 million of commercial vehicle insurance.

          - PSC shall provide at least 4 hours of onsite training with its own personnel to the facilities designated persons to properly train them in the operation, maintenance and troubleshooting of the system.

        - The Commissioning Team shall consist of an Owner’s Representative; Manufacture’s representative; Project Manager, Technical Specialist; and the Commissioning Team Leader.

          - The Project Manager, from the Physical Security Contractor, shall provide technical and managerial leadership to all personnel during the installation and setup phases of the equipment. The Project Manager shall be the primary point of contact for the Owner for this specification. The Project Manager shall have a minimum of 5 years of experience in the installation of similar systems.

          - The Technical Specialist shall have a minimum of 5 years of experience in the installation, operation, and testing of all components of barrier systems.  The technical specialist shall be present in the factory during Factory Tests of the barriers.

          - Commissioning Team Leader shall have a minimum of 5 years of experience running tests of barrier systems, preparing and completing reports, and coordinating personnel for such test.

      - DELIVERY, STORAGE AND HANDLING

        - Store equipment in a location protected from the weather, humidity, temperature variation, dirt and dust, or other contaminants.  Store materials on sleepers or pallets and protect from rust and objectionable materials such as dirt, grease or oil.

      - WARRANTY

        - Warranty:  The Manufacture will provide its standard limited 1-yr warranty on parts.

- PRODUCTS.

      - MANUFACTURER

        - Basis-of-Design Manufacturers:

          - Heald Ltd., Northfield, Atwick Road, Hornsea, East Yorkshire  HU18 1EL.  Telephone: +44 (0)1964 535858.  http://www.heald.uk.com

          - No substitutions will be accepted.

      - SLIDING BOLLARDS

        - Sliding Surface/Shallow-Mount Bollards: Heald MATADOR 6 by Heald Ltd complying with the following:

          - Performance:  Crash tested as a single unit in accordance with IWA14-1:2003 Bollard V/7200[N2A]/64/90:0.1.  Vehicle weight: 15,873 pounds, impact speed 40 mph, impact angle 90 degrees, maximum dynamic penetration 0.1m (0.3 feet).

          - Bollard Assembly:  Structural steel housing below grade with a structural steel bollard that slides across the housing to allow passage of authorized vehicles; flush with roadway surface or surface mounted when installed.

          - Concrete Foundation: Grade C50 (Portland Type 1) concrete. Vibrate concrete to fill all voids.

          - Foundation Depth:  150mm (5.9").

          - Bollard Height: 1140mm (44.8").

          - Opening Size:  6846mm (269.5").

          - Repair:  Capability of removing the bollard element for replacement or repair, without replacement of the entire bollard assembly.

          - [Optional] Tray system available to allow inlaying of paving substances, allowing the Matador 6 fit with any aesthetics.

          - Modular fixed bollard units that can be mounted onto the Matador 6 increasing the protection area.

          - Heat trace provided under Matador where sub-zero temperatures occur or are likely to. All heat-tracing applications with continuous exposure (maintain) temperatures from 150°F (65°C) to 300°F (150°C) or intermittent exposure temperatures from 185°F (85°C) to 482°F (250°C) shall use self-regulating cables. A.	Self-regulating heating cable shall vary its power output relative to the temperature of the surface of the pipe or the vessel. The cable shall be designed such that it can be crossed over itself and cut to length in the field. B.	Self-regulating heating cable shall be designed for a useful life of 20 years or more with “power on” continuously, based on the following useful life criteria: 1.	Retention of at least 75 percent of nominal rated power after 20 years of operation at the maximum published continuous exposure (maintain) temperature.  2.	Retention of at least 90 percent of nominal rated power after 1000 hours of operation at the maximum published intermittent exposure temperature. The testing shall conform to UL 746B, IEC 60216-1.

          - Manual Operation: Open and Close the bollard element manually and lock into position.

        - Finish:

          - Hot-dip galvanized per BS EN ISO 1461 (ASTM A123).

          - [Option] Wash primer after hot-dip galvanizing.

          - [Option] Variety of Bollard covers available.

      - FIXED BOLARDS

        - Shallow-Mount Bollards: Heald MATADOR 6 by Heald Ltd complying with the following:

          - Performance:  Crash tested as a single unit in accordance with IWA14-1:2003 Bollard V/7200[N2A]/64/90:0.1.  Vehicle weight: 15,873 pounds, impact speed 40 mph, impact angle 90 degrees, maximum dynamic penetration 0.1m (0.3 feet).

          - Bollard Assembly:  Structural steel housing below grade with a structural steel bollard flush with roadway surface when installed.

          - Concrete Foundation: Grade C50 (Portland Type 1) concrete. Vibrate concrete to fill all voids.

          - Foundation Depth:  150mm (5.9").

          - Bollard Height: 1140mm (44.8").

- EXECUTION

      - EXAMINATION

        - Examine substrates and site conditions for compliance with requirements for installation tolerances and other conditions affecting performance.  Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install all components and ancillary devices necessary to ensure that the bollard performance is per the approved sequence of operation.

        - Install products in strict accordance with manufacturer’s instructions and approved submittals.  Locate units level, plumb, and in proper alignment with adjacent work. Use concealed anchorages where possible.

          - Protect adjacent areas against damage; repair or patch damaged areas.  Restore damaged finishes so no evidence remains of corrective work.

        - Upon completion of construction, perform a field test for each vehicle bollard. The test shall include opening and closing the bollard manually, through its complete range of operation.

        - A certified manufacturer’s integrator must do the installation or oversee the install for the Matador’s certifications to be valid.

        - [Option] Provide a field-training course for designated operating staff members by the installer. Field training shall cover all the items contained in the operating and maintenance instructions.

      - ADJUSTING AND CLEANING

        - Clean exposed surfaces using methods acceptable to the manufacturer which will not damage finish.

      - PREVENTIVE MAINTENANCE

        - Perform quarterly preventive maintenance to include: visual checks and operational tests of vehicle barriers (cleaning pit if necessary), and peripheral equipment. Though most major tasks will only need to be performed quarterly, operation of the barrier on a daily or weekly basis, as well as occasional cleaning, may be required by owner’s personnel due to environmental conditions. On-site records containing the performed tasks must be kept to validate the equipment warranty.  Required preventive maintenance items shall be covered in the training and listed in the manufacturer’s operation and maintenance manual.

END OF SECTION

NOTE: Please reach out to us for assistance with Matador 4 or other versions as well as to develop the controls scope and specifications.